📌 Application Layer, Port Numbers & DNS

- Communication starts at the Application Layer where every service is assigned a port number.
Examples:

  - HTTP → 80

  - HTTPS → 443

  - DNS → 53

  - FTP → 20/21

  - SMTP → 25

- When a user types a domain name (e.g., google.com), the system contacts the DNS server to convert the domain name → IP address.

- After the IP is obtained, the packet moves to the Network Layer, where routing decisions are made based on the destination IP.

📌 ARP (Address Resolution Protocol)

- ARP is used to find the MAC address corresponding to an IP address.

- Operates at the Network Layer (works between Layer 2 and Layer 3).

Case 1: Destination is in the Same Subnet

- Host checks if the destination IP belongs to its own network.

- If YES → ARP request is broadcast to find the destination host’s MAC address.

- ARP reply is unicast from the destination host.

Case 2: Destination is in a Different Subnet

- Host cannot directly reach the destination.

- ARP request is sent to find MAC address of the Default Gateway (router).

- Packet is delivered to the router → router forwards it to the correct network.

📌 Difference Between ARP and RARP

| ARP | RARP |
| --- | --- |
| Resolves IP → MAC | Resolves MAC → IP |
| Used by normal hosts during communication. | Used by diskless machines that only know their MAC. |
| Sends ARP Request broadcast to find MAC. | Sends RARP Request broadcast to find IP. |
| Part of IPv4 protocol suite. | Mostly obsolete, replaced by BOOTP/DHCP. |
| Helps deliver packets inside LAN. | Helps assign an IP to a host. |

📌 DHCP (Dynamic Host Configuration Protocol)

- DHCP automatically assigns IP address and other configuration parameters to hosts.

- Operates on UDP port 67 (server) and 68 (client).

- Provides:

  - IP address

  - Subnet mask

  - Default gateway

  - DNS server(s)

  - Lease time

📌 Methods of IP Allocation in DHCP

- Automatic Allocation

  - Assigns a permanent IP to a host.

  - Once assigned, the same IP is always given.

- Dynamic Allocation

  - Assigns IP for a limited time (lease).

  - Most commonly used method.

- Manual/Static Allocation (MAC Binding)

  - Admin binds a host’s MAC to a specific IP.

  - DHCP always gives the same IP to that device.

📌 DHCP Operation (DORA Process)

DHCP works in 4 steps called DORA:

1️⃣ DHCP Discover

- Client broadcasts DHCP Discover to find available servers.

- Source: 0.0.0.0

- Destination: 255.255.255.255

2️⃣ DHCP Offer

- DHCP server responds with an Offer containing:

  - Offered IP

  - Subnet mask

  - Lease time

  - Gateway

  - DNS

3️⃣ DHCP Request

- Client sends a broadcast Request confirming it wants the offered IP.

4️⃣ DHCP Acknowledgement

- Server sends DHCP ACK → configuration is finalized.

- Host now starts using the IP.

ICMP :

- ICMP stands for Internet Control Message Protocol.

- It is a network layer protocol used mainly for error reporting and diagnostics.

- When a router or host detects a problem while forwarding an IP packet, it generates an ICMP error message and sends it back to the original sender’s IP address.

Internet Protocol – IPV4 :

- Internet Protocol (IP) is a network layer (Layer 3) protocol responsible for logical addressing and routing packets across networks so they can travel from a source host to a destination host.

✔️ Characteristics of IP :

- Connectionless → No handshake, packets are independent.

- Unreliable → No acknowledgements.

- Stateless → Routers don’t store session info.

- Best-effort → No guarantees, but efficient.

IMP for Numericals :

- HLEN = ( x * 4 )bytes

- TOTAL LEN =  x bytes

| Field Name | Size | Purpose / Function |
| --- | --- | --- |
| Version (VER) | 4 bits | Identifies IP version (IPv4 = 4). |
| Header Length (IHL/HLEN) | 4 bits | Specifies header size (20–60 bytes). Tells where data begins. |
| Type of Service (ToS) / DSCP | 8 bits | Specifies priority/QoS: low delay, high throughput, etc. |
| Total Length | 16 bits | Size of entire packet (header + data). Range: 20–65,535 bytes. |
| Identification | 16 bits | Used for fragmentation; all fragments of a packet share this ID. |
| Flags | 3 bits | Controls fragmentation: DF (Don’t Fragment), MF (More Fragments). |
| Fragment Offset | 13 bits | Indicates fragment’s position in original datagram; used in reassembly. |
| Time To Live (TTL) | 8 bits | Limits packet lifetime; decremented by each router. Prevents loops. |
| Protocol | 8 bits | Indicates encapsulated protocol: ICMP(1), TCP(6), UDP(17). |
| Header Checksum | 16 bits | Error checking for header only; routers verify this. |
| Source IP Address | 32 bits | IP address of sender. |
| Destination IP Address | 32 bits | IP address of receiver; used for routing. |
| Options | 0–40 bytes | Rarely used fields: security, timestamp, record route, etc. |
| Padding | Variable | Added to make header length a multiple of 32 bits. |
| Data (Payload) | Variable | Actual content (TCP, UDP, ICMP, etc.) carried by IP. |

Routing :

Fragmentation :

- Fragmentation is the process of breaking a large IP packet into smaller packets (fragments) so that they can pass through a network whose Maximum Transmission Unit (MTU) is smaller than the packet size.

- It is required because different networks have different MTU values, and a packet larger than the MTU cannot be transmitted as a whole.

IPv4 Header Fields Involved in Fragmentation (with purpose) :

1. Identification (16 bits)

- A unique number assigned by the sender to each packet.

- Purpose:

  - All fragments of the same original packet carry the same Identification number.

  - Helps the destination host reassemble fragments correctly.

2. Flags (3 bits)

Contains 3 bits: Reserved | DF | MF

- DF (Don't Fragment) bit

  - 1 = do not fragment.

  - 0 = fragmentation allowed.

- MF (More Fragments) bit

  - 1 = more fragments are coming.

  - 0 = this is the last fragment.

Purpose:
Controls whether a packet can be fragmented and indicates if more fragments follow.

3. Fragment Offset (13 bits)

- Indicates the position of the fragment in the original packet.

- Value is in units of 8 bytes (because fragment offset counts 8-byte blocks).

- Purpose:

  - Helps destination reassemble fragments in correct order.

4. Total Length Field

- Shows total length (header + data) of the fragment.

- Purpose:

  - Indicates how big each fragment is.

What is a Router?

- A router is a network layer device that forwards packets from one network to another.

- It uses IP addresses and a routing table to decide where to send packets.

- It connects multiple networks (LAN–LAN, LAN–WAN).

Routing

- Routing is the process of selecting the best path for a packet from source to destination.

- It involves running routing algorithms and building/updating the routing table.

Forwarding

- Forwarding is the process of sending a packet to the correct outgoing interface based on the routing table.

- It is a local, per-packet operation inside a router.

Routing vs Forwarding (Differences)

| Routing | Forwarding |
| --- | --- |
| Path selection process. | Actual movement of packets. |
| Creates/updates the routing table. | Uses the routing table to pick the next hop. |
| Done by routing protocols (RIP, OSPF, BGP). | Done by router hardware quickly. |
| Network-wide decision. | Local router decision. |
| Happens occasionally. | Happens for every packet. |

Static Routing

- Routes are manually configured by the network admin.

- Do not change automatically if network topology changes.

- Best for small, stable networks.

Dynamic Routing

- Routes are learned automatically using routing protocols.

- Routing tables update whenever topology changes.

- Best for large, changing networks.

Static vs Dynamic Routing (Exam Table)

| Static Routing | Dynamic Routing |
| --- | --- |
| Manually configured. | Learned automatically. |
| No control traffic. | Uses control packets. |
| No CPU overhead. | Higher CPU and memory usage. |
| Not scalable. | Highly scalable. |
| Does NOT adapt automatically. | Adapts to failures automatically. |

Dynamic Routing Protocols

Two types:

1. Distance Vector Protocols

- Routers share routing tables with neighbors.

- Decision based on distance metric (hop count).

- Examples:

  - RIP (Routing Information Protocol)

  - IGRP

2. Link State Protocols

- Routers flood complete topology information in the network.

- Each router runs Dijkstra’s shortest path algorithm.

- Examples:

  - OSPF (Open Shortest Path First)

  - IS-IS

Distance Vector Protocol (RIP)

- Each router maintains a routing table of distance (hop count) to all networks.

- Periodically shares this table with neighbor routers.

- Based on Bellman–Ford algorithm.

- Maximum hop count = 15, 16 = infinity.

Count-to-Infinity Problem

- In distance vector routing, when a link fails, routers may incorrectly keep increasing hop count for a network, taking a long time to converge.

- They “count” slowly upward until they reach infinity (16).

- This happens because routers trust their neighbors blindly.

Solutions to Count-to-Infinity

1. Split Horizon

- A router does not advertise a route back on the same interface from which it learned it.

Meaning:
If Router A learned about Network N from Router B,
→ A will not send the information about N back to Router B.

This prevents routing loops.

2. Poisoned Reverse (Poisoning)

- Router advertises the route back to the neighbor with metric = infinity (16 in RIP).

- Tells the neighbor:
"Don’t use me for this route — I cannot reach it."

This breaks loops faster than split horizon.

RIP (Routing Information Protocol)

RIP is a distance-vector dynamic routing protocol used in small to medium networks.

Key Features

- Uses distance vector routing based on hop count.

- Maximum hop count = 15 (16 is infinity → unreachable).

- Uses UDP port 520.

- Uses the Bellman–Ford algorithm.

- Periodic routing updates to neighbors only.

Timers in RIP :

| Timer Name | Default Value | Meaning / What it Does | Why It Is Used |
| --- | --- | --- | --- |
| Update Timer | 30 seconds | Router broadcasts its entire routing table to all neighbors every 30 seconds. | Ensures all routers stay updated with the latest routing information. |
| Invalid Timer | 180 seconds | If no update is received for a route for 180 seconds, that route is marked invalid and metric is set to 16 (infinity). | Detects route failures; decides how long a route can stay without being refreshed. |
| Hold-Down Timer | 180 seconds | After a route becomes invalid or shows a worse metric, router enters hold-down state where it ignores all updates for that route unless a better metric is advertised. | Prevents routing loops and unstable fluctuations after a failure. |
| Flush Timer | 240 seconds | After a route is marked invalid, router waits until 240 seconds to completely remove the route from the table. | Gives routers enough time to propagate failure info before deleting the entry. |

RIP messages :

1.  Request Message : request for RIP routing table.

2. Response Message : respond with RIP routing table.

Link state protocol continue after computers doc :

We know it comprises of two phases :

    - Reliable transfer

    - Shortest path : For this it uses Dijkstra’s algorithm.

      - Maintains tentaive list.

      - Maintains confirmed list.

      - Both maintain an entry of form (Destination,cost,nextHop)